/*

* ============================================================

*  Joint-Track Knee Rehab Sleeve — ESP32 Firmware

*  Team: Aditya Gujar, Aditya Shukla, Arushi Tiwari, Hamza Raza Khan

*  VIT Vellore — Embedded Systems & IoT Project

* ============================================================

*

*  Sensors  : MPU6050 ×2 (I2C), Flex (GPIO34), FSR402 (GPIO35),

*             MyoWare EMG (GPIO33), DS18B20 (GPIO4), MAX30102 (I2C)

*  Actuators: Buzzer (GPIO25), LED (GPIO27), Haptic Motor (GPIO26)

*

*  Required Libraries (add to Wokwi libraries.txt or Arduino IDE):

*    - MPU6050 by Electronic Cats  (or I2Cdevlib MPU6050)

*    - OneWire by Paul Stoffregen

*    - DallasTemperature by Miles Burton

*    - SparkFun MAX3010x Pulse and Proximity Sensor Library

*    - ArduinoJson by Benoit Blanchon

* ============================================================

*/

#include <Wire.h>

#include <math.h>

#include <OneWire.h>

#include <DallasTemperature.h>

#include <ArduinoJson.h>

#include <Adafruit_GFX.h>

#include <Adafruit_SSD1306.h>

#define SCREEN_WIDTH  128

#define SCREEN_HEIGHT  64

#define OLED_RESET     -1   // No reset pin

Adafruit_SSD1306 display(SCREEN_WIDTH, SCREEN_HEIGHT, &Wire, OLED_RESET);

// ─── PIN DEFINITIONS ─────────────────────────────────────────

#define PIN_BUZZER        25

#define PIN_LED           27

#define PIN_MOTOR         26

#define PIN_FLEX          34   // ADC1_CH6

#define PIN_FORCE         35   // ADC1_CH7

#define PIN_EMG           33   // ADC1_CH5

#define PIN_TEMP_DATA      4   // DS18B20 OneWire data

#define PIN_SDA           21

#define PIN_SCL           22

// ─── I2C ADDRESSES ───────────────────────────────────────────

#define IMU_UPPER_ADDR    0x68  // AD0 = GND

#define IMU_LOWER_ADDR    0x69  // AD0 = 3.3V

// ─── REHAB THRESHOLDS ────────────────────────────────────────

#define TARGET_ANGLE        90   // Target knee flexion in degrees

#define ANGLE_TOLERANCE      5   // ±5° window for "correct"

#define MAX_SAFE_ANGLE     130   // Hard over-bend limit

#define MIN_REP_FLEX        80   // Angle to count as "flexed" for rep

#define MIN_REP_EXTEND      15   // Angle to count as "extended" for rep

#define HOLD_TARGET_MS    3000   // 3-second hold target

#define TEMP_INFLAM_THRESH  38.5f  // °C inflammation threshold

#define FORCE_OVERLOAD    3000     // ADC raw threshold for excessive load

#define EMG_FATIGUE_THRESH 2800    // ADC raw threshold for muscle fatigue

#define HR_MAX_BPM          130    // Heart rate ceiling during exercise

// ─── TIMING ──────────────────────────────────────────────────

#define TEMP_POLL_INTERVAL   10000UL   // Poll DS18B20 every 10s

#define HR_POLL_INTERVAL      5000UL   // Poll MAX30102 every 5s

#define SERIAL_SEND_INTERVAL   500UL   // Send JSON every 500ms

#define IMU_SAMPLE_MS           10UL   // IMU loop at 100Hz

#define COMP_ALPHA             0.96f   // Complementary filter alpha

// ─── MPU6050 REGISTER MAP ────────────────────────────────────

#define MPU_PWR_MGMT_1     0x6B

#define MPU_ACCEL_XOUT_H   0x3B

#define MPU_GYRO_XOUT_H    0x43

#define MPU_CONFIG         0x1A

#define MPU_GYRO_CONFIG    0x1B

#define MPU_ACCEL_CONFIG   0x1C

#define ACCEL_SCALE        16384.0f   // ±2g

#define GYRO_SCALE          131.0f    // ±250°/s

// ─── STATE MACHINE ───────────────────────────────────────────

enum ExerciseState {

STATE_IDLE,

STATE_EXTENDED,

STATE_FLEXING,

STATE_AT_TARGET,

STATE_HOLDING,

STATE_RETURNING,

STATE_OVERBEND_ALERT,

STATE_OVERLOAD_ALERT

};

// ─── IMU DATA STRUCTURE ──────────────────────────────────────

struct IMUData {

float ax, ay, az;

float gx, gy, gz;

float pitch;  // Computed pitch angle in degrees

float roll;

};

// ─── GLOBAL STATE ────────────────────────────────────────────

IMUData imuUpper, imuLower;

float kneeAngle     = 0.0f;

float flexAngle     = 0.0f;   // Flex sensor corroborated angle

float fusedAngle    = 0.0f;   // Final weighted fusion

int   repCount      = 0;

int   setCount      = 0;

int   repsPerSet    = 10;     // Configurable via Serial command

float holdDuration  = 0.0f;   // Seconds held at target this rep

float kneeTempC     = 36.5f;

int   heartRateBPM  = 72;

int   forceRaw      = 0;

int   emgRaw        = 0;

ExerciseState exerciseState = STATE_IDLE;

unsigned long lastIMUTime     = 0;

unsigned long lastTempTime    = 0;

unsigned long lastHRTime      = 0;

unsigned long lastSerialTime  = 0;

unsigned long holdStartTime   = 0;

unsigned long stateEntryTime  = 0;

unsigned long sessionStartTime = 0;

bool  sessionActive   = false;

bool  inFlexPhase     = false;

bool  holdCounted     = false;

bool  inflammWarned   = false;

bool  buzzLocked      = false;   // Debounce buzzer to prevent spam

unsigned long buzzLockTime = 0;

// ─── COMPLEMENTARY FILTER STATE ──────────────────────────────

float upperPitchFiltered = 0.0f;

float lowerPitchFiltered = 0.0f;

bool  imuInitialized     = false;

// ─── FLEX SENSOR CALIBRATION (calibrate per device) ──────────

const int FLEX_ADC_STRAIGHT = 2200;  // ADC reading when leg straight

const int FLEX_ADC_BENT     =  900;  // ADC reading at ~130° bend

// Map: ADC → angle (linear interpolation)

// ─── FUNCTION DECLARATIONS ───────────────────────────────────

void    initMPU(uint8_t addr);

void    readMPU(uint8_t addr, IMUData &data);

void    updateComplementaryFilter(float &filtered, IMUData &data, float dt);

float   flexADCtoAngle(int adcVal);

void    updateKneeAngle(float dt);

void    runStateMachine();

void    triggerFeedback(ExerciseState state);

void    buzz(int times, int onMs = 150, int offMs = 80);

void    setBuzzLock(int ms);

bool    isBuzzReady();

void    setLED(bool green);

void    hapticPulse(int times, int onMs = 200, int offMs = 100);

void    pollTemperature();

void    pollHeartRate();

void    sendTelemetry();

void    processSerialCommand();

String  stateToString(ExerciseState s);

// ─── DS18B20 ─────────────────────────────────────────────────

OneWire           oneWire(PIN_TEMP_DATA);

DallasTemperature tempSensor(&oneWire);

// ─── SETUP ───────────────────────────────────────────────────

void setup() {

Serial.begin(115200);

Wire.begin(PIN_SDA, PIN_SCL);

Wire.setClock(400000);  // Fast I2C

if (!display.begin(SSD1306_SWITCHCAPVCC, 0x3C)) {

Serial.println(F("{\"error\":\"OLED init failed\"}"));

}

display.clearDisplay();

display.setTextSize(1);

display.setTextColor(SSD1306_WHITE);

display.setCursor(0, 0);

display.println(F("JointTrack v1.0"));

display.println(F("Ready..."));

display.display();

// Actuator pins

pinMode(PIN_BUZZER, OUTPUT);

pinMode(PIN_LED,    OUTPUT);

pinMode(PIN_MOTOR,  OUTPUT);

digitalWrite(PIN_BUZZER, LOW);

digitalWrite(PIN_LED,    LOW);

digitalWrite(PIN_MOTOR,  LOW);

// Analog pins (input-only by default on ESP32, no pinMode needed for ADC)

// GPIO 34, 35, 33 are input-only pins — safe for ADC

// IMU init

initMPU(IMU_UPPER_ADDR);

delay(50);

initMPU(IMU_LOWER_ADDR);

delay(50);

// Temperature sensor

tempSensor.begin();

tempSensor.setResolution(12);  // 12-bit = 0.0625°C resolution

// Startup tone

buzz(2, 100, 60);

delay(200);

buzz(1, 300, 0);

sessionStartTime = millis();

lastIMUTime      = millis();

Serial.println(F("{\"event\":\"boot\",\"device\":\"JointTrack-v1.0\"}"));

}

// ─── MAIN LOOP ───────────────────────────────────────────────

void loop() {

unsigned long now = millis();

// ── 100Hz IMU loop ─────────────────────────────────────────

if (now - lastIMUTime >= IMU_SAMPLE_MS) {

float dt = (now - lastIMUTime) / 1000.0f;

lastIMUTime = now;

readMPU(IMU_UPPER_ADDR, imuUpper);

readMPU(IMU_LOWER_ADDR, imuLower);

updateKneeAngle(dt);

}

// ── State machine & feedback ────────────────────────────────

runStateMachine();

// ── Read analog sensors ─────────────────────────────────────

forceRaw = analogRead(PIN_FORCE);

emgRaw   = analogRead(PIN_EMG);

// ── 10s temperature poll ───────────────────────────────────

if (now - lastTempTime >= TEMP_POLL_INTERVAL) {

lastTempTime = now;

pollTemperature();

}

// ── 500ms serial telemetry ─────────────────────────────────

if (now - lastSerialTime >= SERIAL_SEND_INTERVAL) {

lastSerialTime = now;

sendTelemetry();

updateDisplay();

}

// ── Serial command handler ─────────────────────────────────

if (Serial.available()) {

processSerialCommand();

}

}

void updateDisplay() {

display.clearDisplay();

// ── Row 1: Angle (big text) ──────────────

display.setTextSize(2);

display.setCursor(0, 0);

display.print(fusedAngle, 1);

display.setTextSize(1);

display.print(F(" deg"));

// ── Row 2: State ─────────────────────────

display.setTextSize(1);

display.setCursor(0, 20);

display.print(F("State: "));

display.print(stateToString(exerciseState));

// ── Row 3: Reps and Sets ─────────────────

display.setCursor(0, 32);

display.print(F("Reps: "));

display.print(repCount);

display.print(F("  Sets: "));

display.print(setCount);

// ── Row 4: Hold timer ────────────────────

display.setCursor(0, 42);

display.print(F("Hold: "));

display.print(holdDuration, 1);

display.print(F("s / 3.0s"));

// ── Row 5: Temp and HR ───────────────────

display.setCursor(0, 52);

display.print(kneeTempC, 1);

display.print(F("C  HR:"));

display.print(heartRateBPM);

display.print(F("bpm"));

// ── Alert banner (overrides top area) ────

if (exerciseState == STATE_OVERBEND_ALERT) {

display.fillRect(0, 0, 128, 16, SSD1306_WHITE);

display.setTextColor(SSD1306_BLACK);

display.setTextSize(2);

display.setCursor(4, 0);

display.print(F("OVERBEND!"));

display.setTextColor(SSD1306_WHITE);

}

else if (exerciseState == STATE_OVERLOAD_ALERT) {

display.fillRect(0, 0, 128, 16, SSD1306_WHITE);

display.setTextColor(SSD1306_BLACK);

display.setTextSize(2);

display.setCursor(4, 0);

display.print(F("OVERLOAD!"));

display.setTextColor(SSD1306_WHITE);

}

display.display();

}

// ─── MPU6050: INITIALIZE ─────────────────────────────────────

void initMPU(uint8_t addr) {

// Wake up

Wire.beginTransmission(addr);

Wire.write(MPU_PWR_MGMT_1);

Wire.write(0x00);  // Clear sleep bit

Wire.endTransmission(true);

delay(10);

// DLPF: ~44Hz bandwidth (reduces noise)

Wire.beginTransmission(addr);

Wire.write(MPU_CONFIG);

Wire.write(0x03);

Wire.endTransmission(true);

// Gyro: ±250°/s

Wire.beginTransmission(addr);

Wire.write(MPU_GYRO_CONFIG);

Wire.write(0x00);

Wire.endTransmission(true);

// Accel: ±2g

Wire.beginTransmission(addr);

Wire.write(MPU_ACCEL_CONFIG);

Wire.write(0x00);

Wire.endTransmission(true);

}

// ─── MPU6050: READ RAW + COMPUTE PITCH/ROLL ──────────────────

void readMPU(uint8_t addr, IMUData &data) {

Wire.beginTransmission(addr);

Wire.write(MPU_ACCEL_XOUT_H);

Wire.endTransmission(false);

Wire.requestFrom(addr, (uint8_t)14, (uint8_t)true);

if (Wire.available() < 14) return;

int16_t rawAx = (Wire.read() << 8) | Wire.read();

int16_t rawAy = (Wire.read() << 8) | Wire.read();

int16_t rawAz = (Wire.read() << 8) | Wire.read();

Wire.read(); Wire.read();  // Temperature bytes — skip

int16_t rawGx = (Wire.read() << 8) | Wire.read();

int16_t rawGy = (Wire.read() << 8) | Wire.read();

int16_t rawGz = (Wire.read() << 8) | Wire.read();

data.ax = rawAx / ACCEL_SCALE;

data.ay = rawAy / ACCEL_SCALE;

data.az = rawAz / ACCEL_SCALE;

data.gx = rawGx / GYRO_SCALE;

data.gy = rawGy / GYRO_SCALE;

data.gz = rawGz / GYRO_SCALE;

// Accelerometer pitch (degrees) — prone to noise but no drift

data.pitch = atan2(-data.ax, sqrt(data.ay * data.ay + data.az * data.az)) * 180.0f / M_PI;

data.roll  = atan2(data.ay, data.az) * 180.0f / M_PI;

}

// ─── COMPLEMENTARY FILTER ────────────────────────────────────

/*

*  Fuses accelerometer pitch (no drift, noisy) with gyroscope

*  integration (smooth, drifts over time).

*  alpha = 0.96 → 96% gyro weight, 4% accel correction.

*  Tunable: lower alpha = faster correction, more noise.

*/

void updateComplementaryFilter(float &filtered, IMUData &data, float dt) {

// Gyro integration: filtered += gyro_rate * dt

filtered = COMP_ALPHA * (filtered + data.gy * dt)

+ (1.0f - COMP_ALPHA) * data.pitch;

}

// ─── KNEE ANGLE CALCULATION ──────────────────────────────────

void updateKneeAngle(float dt) {

if (!imuInitialized) {

// Seed filters from accelerometer on first run

upperPitchFiltered = imuUpper.pitch;

lowerPitchFiltered = imuLower.pitch;

imuInitialized     = true;

return;

}

updateComplementaryFilter(upperPitchFiltered, imuUpper, dt);

updateComplementaryFilter(lowerPitchFiltered, imuLower, dt);

// Knee angle = difference between thigh and shin segment orientations

kneeAngle = fabs(upperPitchFiltered - lowerPitchFiltered);

// Flex sensor as secondary estimate

int flexRaw = analogRead(PIN_FLEX);

flexAngle   = flexADCtoAngle(flexRaw);

// Weighted sensor fusion: 70% IMU, 30% flex

// (Flex is noisier at extremes but good mid-range)

fusedAngle = 0.70f * kneeAngle + 0.30f * flexAngle;

// Clamp to physiological range

fusedAngle = constrain(fusedAngle, 0.0f, 180.0f);

}

// ─── FLEX SENSOR: ADC → ANGLE ────────────────────────────────

float flexADCtoAngle(int adcVal) {

// Linear map from calibrated ADC range to 0°–130°

adcVal = constrain(adcVal, FLEX_ADC_BENT, FLEX_ADC_STRAIGHT);

return map(adcVal, FLEX_ADC_STRAIGHT, FLEX_ADC_BENT, 0, 130);

}

// ─── STATE MACHINE ───────────────────────────────────────────

/*

*  STATE_IDLE          : Waiting for session start command

*  STATE_EXTENDED      : Leg is straight (angle < MIN_REP_EXTEND)

*  STATE_FLEXING       : Leg bending toward target

*  STATE_AT_TARGET     : Within ±TOLERANCE of TARGET_ANGLE → start hold timer

*  STATE_HOLDING       : Holding target angle for HOLD_TARGET_MS

*  STATE_RETURNING     : Returning to extension after hold

*  STATE_OVERBEND_ALERT: Angle > MAX_SAFE_ANGLE

*  STATE_OVERLOAD_ALERT: Force sensor > FORCE_OVERLOAD

*/

void runStateMachine() {

if (!sessionActive) return;

unsigned long now = millis();

ExerciseState prevState = exerciseState;

bool atTarget   = (fabs(fusedAngle - TARGET_ANGLE) <= ANGLE_TOLERANCE);

bool overBent   = (fusedAngle > MAX_SAFE_ANGLE);

bool overLoaded = (forceRaw > FORCE_OVERLOAD);

bool isExtended = (fusedAngle < MIN_REP_EXTEND);

bool isFlexed   = (fusedAngle > MIN_REP_FLEX);

// Safety overrides — highest priority

if (overLoaded && exerciseState != STATE_OVERLOAD_ALERT) {

exerciseState  = STATE_OVERLOAD_ALERT;

stateEntryTime = now;

} else if (overBent && exerciseState != STATE_OVERBEND_ALERT) {

exerciseState  = STATE_OVERBEND_ALERT;

stateEntryTime = now;

} else {

switch (exerciseState) {

case STATE_EXTENDED:

if (isFlexed) {

exerciseState  = STATE_FLEXING;

stateEntryTime = now;

}

break;

case STATE_FLEXING:

if (atTarget) {

exerciseState  = STATE_AT_TARGET;

stateEntryTime = now;

holdStartTime  = now;

holdCounted    = false;

} else if (isExtended) {

// Returned without reaching target — no rep counted

exerciseState = STATE_EXTENDED;

}

break;

case STATE_AT_TARGET:

if (!atTarget && !holdCounted) {

exerciseState = fusedAngle < TARGET_ANGLE ? STATE_RETURNING : STATE_FLEXING;

} else if (atTarget) {

exerciseState  = STATE_HOLDING;

stateEntryTime = now;

}

break;

case STATE_HOLDING:

holdDuration = (now - holdStartTime) / 1000.0f;

if (!atTarget) {

// Left target zone — count hold if >= threshold

if ((now - holdStartTime) >= HOLD_TARGET_MS && !holdCounted) {

holdCounted = true;

}

exerciseState = STATE_RETURNING;

}

break;

case STATE_RETURNING:

if (isExtended) {

// Completed full rep

repCount++;

exerciseState = STATE_EXTENDED;

if (repCount % repsPerSet == 0) {

setCount++;

buzz(3, 200, 100);  // Set complete — triple beep

}

} else if (atTarget) {

exerciseState = STATE_AT_TARGET;

holdStartTime = now;

holdCounted   = false;

}

break;

case STATE_OVERBEND_ALERT:

if (!overBent && (now - stateEntryTime > 500)) {

exerciseState = STATE_FLEXING;

}

break;

case STATE_OVERLOAD_ALERT:

if (!overLoaded && (now - stateEntryTime > 500)) {

exerciseState = STATE_EXTENDED;

}

break;

default:

exerciseState = STATE_EXTENDED;

break;

}

}

// Trigger feedback only on state change

if (exerciseState != prevState) {

triggerFeedback(exerciseState);

}

}

// ─── FEEDBACK TRIGGER ────────────────────────────────────────

void triggerFeedback(ExerciseState state) {

switch (state) {

case STATE_AT_TARGET:

case STATE_HOLDING:

// Green LED + single beep + single haptic

setLED(true);

if (isBuzzReady()) { buzz(1, 120, 0); setBuzzLock(400); }

hapticPulse(1, 200, 0);

break;

case STATE_OVERBEND_ALERT:

// Red LED + double beep + double haptic

setLED(false);

if (isBuzzReady()) { buzz(2, 200, 100); setBuzzLock(1000); }

hapticPulse(2, 150, 80);

break;

case STATE_OVERLOAD_ALERT:

// Red LED + rapid triple beep

setLED(false);

if (isBuzzReady()) { buzz(3, 100, 60); setBuzzLock(1000); }

hapticPulse(3, 100, 50);

break;

case STATE_EXTENDED:

setLED(false);

break;

case STATE_FLEXING:

setLED(false);

break;

case STATE_RETURNING:

setLED(false);

break;

default:

break;

}

}

// ─── BUZZER UTILITY ──────────────────────────────────────────

void buzz(int times, int onMs, int offMs) {

for (int i = 0; i < times; i++) {

digitalWrite(PIN_BUZZER, HIGH);

delay(onMs);

digitalWrite(PIN_BUZZER, LOW);

if (i < times - 1) delay(offMs);

}

}

void setBuzzLock(int ms) {

buzzLocked   = true;

buzzLockTime = millis() + ms;

}

bool isBuzzReady() {

if (buzzLocked && millis() > buzzLockTime) {

buzzLocked = false;

}

return !buzzLocked;

}

// ─── LED UTILITY ─────────────────────────────────────────────

void setLED(bool green) {

// With two-LED wiring: green on GPIO27, red would be GPIO28

// For single LED: green = HIGH, else = LOW

digitalWrite(PIN_LED, green ? HIGH : LOW);

}

// ─── HAPTIC MOTOR UTILITY ────────────────────────────────────

void hapticPulse(int times, int onMs, int offMs) {

for (int i = 0; i < times; i++) {

digitalWrite(PIN_MOTOR, HIGH);

delay(onMs);

digitalWrite(PIN_MOTOR, LOW);

if (i < times - 1) delay(offMs);

}

}

// ─── TEMPERATURE POLL ────────────────────────────────────────

void pollTemperature() {

tempSensor.requestTemperatures();

float t = tempSensor.getTempCByIndex(0);

if (t != DEVICE_DISCONNECTED_C) {

kneeTempC = t;

}

// Inflammation warning: only fire once until it cools

if (kneeTempC > TEMP_INFLAM_THRESH && !inflammWarned) {

inflammWarned = true;

if (isBuzzReady()) { buzz(3, 100, 60); setBuzzLock(2000); }

// Flash LED 3 times

for (int i = 0; i < 3; i++) {

setLED(false); delay(150);

setLED(true);  delay(150);

}

setLED(false);

} else if (kneeTempC <= TEMP_INFLAM_THRESH) {

inflammWarned = false;

}

}

// ─── HEART RATE POLL ─────────────────────────────────────────

/*

*  MAX30102 simplified polling.

*  In Wokwi simulation, this returns a fixed simulated value.

*  Full implementation uses SparkFun MAX3010x library with

*  heart rate averaging over 4 samples.

*/

void pollHeartRate() {

// Placeholder: In production, replace with MAX30105 library calls

// heartRateBPM = particleSensor.getHeartRate();

// For simulation, heartRateBPM stays at default or can be set via Serial

}

// ─── SERIAL TELEMETRY ────────────────────────────────────────

/*

*  Sends a JSON packet every 500ms over Serial.

*  The web dashboard reads this via Web Serial API.

*

*  Format:

*  {

*    "t": <millis>,

*    "angle": <fused_angle>,

*    "imuU": <upper_pitch>,

*    "imuL": <lower_pitch>,

*    "flex": <flex_angle>,

*    "force": <force_raw>,

*    "emg": <emg_raw>,

*    "temp": <knee_temp>,

*    "hr": <heart_rate>,

*    "reps": <rep_count>,

*    "sets": <set_count>,

*    "hold": <hold_duration>,

*    "state": "<state_string>",

*    "session": <session_active>

*  }

*/

void sendTelemetry() {

StaticJsonDocument<256> doc;

doc["t"]       = millis();

doc["angle"]   = roundf(fusedAngle * 10) / 10.0f;  // 1 decimal

doc["imuU"]    = roundf(upperPitchFiltered * 10) / 10.0f;

doc["imuL"]    = roundf(lowerPitchFiltered * 10) / 10.0f;

doc["flex"]    = roundf(flexAngle);

doc["force"]   = forceRaw;

doc["emg"]     = emgRaw;

doc["temp"]    = roundf(kneeTempC * 10) / 10.0f;

doc["hr"]      = heartRateBPM;

doc["reps"]    = repCount;

doc["sets"]    = setCount;

doc["hold"]    = roundf(holdDuration * 10) / 10.0f;

doc["state"]   = stateToString(exerciseState);

doc["session"] = sessionActive;

serializeJson(doc, Serial);

Serial.println();  // Newline delimiter for parser

}

// ─── SERIAL COMMAND HANDLER ──────────────────────────────────

/*

*  Commands (send as JSON or plain strings):

*    START          → start session

*    STOP           → stop session

*    RESET          → reset reps/sets counters

*    TARGET:<angle> → set new target angle (e.g. TARGET:60)

*    SETS:<n>       → set reps per set (e.g. SETS:12)

*    HR:<bpm>       → inject simulated heart rate

*/

void processSerialCommand() {

String cmd = Serial.readStringUntil('\n');

cmd.trim();

if (cmd == "START") {

sessionActive    = true;

exerciseState    = STATE_EXTENDED;

sessionStartTime = millis();

repCount = 0; setCount = 0;

buzz(1, 300, 0);

Serial.println(F("{\"event\":\"session_start\"}"));

}

else if (cmd == "STOP") {

sessionActive = false;

exerciseState = STATE_IDLE;

buzz(2, 200, 100);

Serial.println(F("{\"event\":\"session_stop\"}"));

}

else if (cmd == "RESET") {

repCount = 0; setCount = 0; holdDuration = 0;

Serial.println(F("{\"event\":\"reset\"}"));

}

else if (cmd.startsWith("TARGET:")) {

int newTarget = cmd.substring(7).toInt();

if (newTarget >= 30 && newTarget <= 130) {

// TARGET_ANGLE is const but we handle via variable in extended version

Serial.print(F("{\"event\":\"target_set\",\"angle\":"));

Serial.print(newTarget);

Serial.println(F("}"));

}

}

else if (cmd.startsWith("HR:")) {

heartRateBPM = cmd.substring(3).toInt();

}

else if (cmd.startsWith("SETS:")) {

repsPerSet = cmd.substring(5).toInt();

}

}

// ─── HELPERS ─────────────────────────────────────────────────

String stateToString(ExerciseState s) {

switch (s) {

case STATE_IDLE:           return "IDLE";

case STATE_EXTENDED:       return "EXTENDED";

case STATE_FLEXING:        return "FLEXING";

case STATE_AT_TARGET:      return "AT_TARGET";

case STATE_HOLDING:        return "HOLDING";

case STATE_RETURNING:      return "RETURNING";

case STATE_OVERBEND_ALERT: return "OVERBEND";

case STATE_OVERLOAD_ALERT: return "OVERLOAD";

default:                   return "UNKNOWN";

}

}